Billet loading table; pusher; billet furnace; billet shear; billet loader and press).

Press: Danieli Breda

Year of construction 1992

Original revolver type: 3 containers positioned at 120°.

Modified to single container in 1994.

Power: 1600T (Nominal pressure 230 bar); increased to 1900T (nominal pressure 250 bar) in 1994

Productivity: 1820Kg net /hour

Reliability: breakdown rate < at 3%

Technical Spec:

- Billet length:     Maximum load length in the Press container: 900mm . Billet length delivered by suppliers: 7000 mm, diameter 7 inches.

- Type of press: Rear loading press (Back loading)

- Main piston dia.: 915 mm

- Side cylinders dia.: Bore 200 mm, rod diameter 140 mm.

- Max. extrusion speed:    33mm/s

- Dead cycle time:  12,5 S

- Container cylinder dimensions: Bore 225 mm, rod diameter 115 mm

- Butt shear cylinder dimensions: Bore 160 mm, rod diameter 125 mm.

- Vertical butt shear with butt ejector  ??? (Yes/No) Pneumatic ejector

- Tooling: How many container 3 container available . One on the press ; two in spare.

- Container bore dia.:    186 mm

- Number of Heating zones up/down Single.  (The heating elements surround the container, positioned between the frame and the container)

- Stem tool quantity 3 parts

- Dummy block bayonnet castool: quantity 3  Heydasch dummy block to screw

- Vertical imerged main variable flow piston pumps: Rexroth or Oilgear – Rexroth: one A2V 500 HS, two A4VSO 500 HS4 (Rexroth) and one auxiliary pump A4VSO 250 DR/10

- 1 container sealing piston pump: RPM A10VSO 18 DFR1 (Rexroth)

- 1 pilot pump: RPM A10VSO 45 DFR1 (Rexroth)

- 1 filtering pump: RPM Rexroth 3 vis, 7 bars, 957l/min

- PLC system:   GE FANUC

Billet furnace:  (Cometal – 2007)

Energy:  Natural gas (PCI = 10.3 kWh/Nm3)

Oxidizer: Hot air at 200°C (heating zones)

Cold air (conical heating zones at the oven outlet)

Hot air:       1 exchanger for the 4 heating zones (Z3 to Z6).

Number of heating zones:    4 zones Z6+Z5+Z4+Z3

Number of conical heating zones: 2 zones Z2+Z1

Number of burners per zone:

- Z6+Z5+Z4+Z3 = 40 burners/zone

- Z2 = 14 burners

- Z1 = 9 burners

Total heating power:    4 X 376 = 1500 kW

Total power conical zone 1: 198 kW

Total power conical zone 2: 127 kW

Billet loading table / Pusher: Cometal

Year of manufacture: 2007

All of this equipment is well maintained and has a good level of reliability.

Concerning the press , several cracks were detected in 2021 during a maintenance checks.

The cracks were repaired in August 2022 by a specialized company (TAQUET).

Since then, we have been planning ultrasonic inspections of welds as well as at several control points every 5 months by the company SCIMEX.

The various checks carried out since 2022 have not noted any anomalies.